GPU Requirements for Real-Time Model Inference

Per-Model VRAM & GPU Usage

| Model Name | Purpose | Approx. VRAM (1 Camera, 1080p) | Recommended GPU | Notes |
| --- | --- | --- | --- | --- |
| yolov12s-face.pt | Face detection | ~2 GB | RTX 3060 (12 GB) | Detects faces before recognition |
| DeepFace (ArcFace) | Face recognition | ~2.5 GB | RTX 3060 Ti (8 GB) / RTX 4060 Ti (16 GB) | Embedding + comparison step |
| yolov8n-pose.pt | Pose estimation | ~2 GB | RTX 3060 (12 GB) | Lightweight pose model |
| yolo11m.pt | HSE (medium) | ~3 GB | RTX 3060 Ti (8 GB) / RTX 4060 Ti (16 GB) | Human Safety Equipment |
| yolo11m.pt | Material tracking (medium) | ~3 GB | RTX 3060 Ti (8 GB) / RTX 4060 Ti (16 GB) | Material Management & Counting |
| yolov8m.pt + OCR | Object detection (medium) | ~3 GB + ~1 GB = 4 GB | RTX 3070 (8 GB) / RTX 4060 Ti (16 GB) | Number plate detection (for vehicle detection) |

Total System Requirements:

| Setup | Total VRAM Needed | Recommended GPU |
| --- | --- | --- |
| All models running together | ≈ 18-22 GB | RTX 4090 (24 GB) /A6000 (48 GB) |

GPU Comparison: RTX 4090 vs A6000 vs RTX 3060 vs RTX 3070

Technical Specifications Comparison

| Specification | RTX 4090 | RTX A6000 | RTX 3060 | RTX 3070 |
| --- | --- | --- | --- | --- |
| Architecture | Ada Lovelace (5nm) | Ampere (8nm) | Ampere (8nm) | Ampere (8nm) |
| CUDA Cores | 16,384 | 10,752 | 3,584 | 5,888 |
| Tensor Cores | 512 (4th Gen) | 336 (3rd Gen) | 112 (3rd Gen) | 184 (3rd Gen) |
| RT Cores | 128 (3rd Gen) | 84 (2nd Gen) | 28 (2nd Gen) | 46 (2nd Gen) |
| VRAM | 24GB GDDR6X | 48GB GDDR6 (ECC) | 12GB GDDR6 | 8GB GDDR6 |
| Memory Bus Width | 384-bit | 384-bit | 192-bit | 256-bit |
| Memory Bandwidth | 1,008 GB/s | 768 GB/s | 360 GB/s | 448 GB/s |
| Base Clock | 2,235 MHz | 1,410 MHz | 1,320 MHz | 1,500 MHz |
| Boost Clock | 2,520 MHz | 1,800 MHz | 1,777 MHz | 1,725 MHz |
| FP32 Performance | ~82.6 TFLOPS | 38.7 TFLOPS | ~12.7 TFLOPS | ~20.3 TFLOPS |
| RT Core Performance | ~191 TFLOPS | 75.6 TFLOPS | ~24.9 TFLOPS | ~40.2 TFLOPS |
| Tensor Performance | ~1,321 TFLOPS | 309.7 TFLOPS | ~101.6 TFLOPS | ~163.1 TFLOPS |
| TDP (Power Draw) | 450W | 300W | 170W | 220W |
| Recommended PSU | 850W+ | 650W+ | 550W+ | 650W+ |
| PCIe Interface | PCIe 4.0 x16 | PCIe 4.0 x16 | PCIe 4.0 x16 | PCIe 4.0 x16 |
| Form Factor | 3-slot | 2-slot (dual-slot) | 2-slot | 2-slot |
| Dimensions (L x H) | 304mm x 137mm | 267mm x 112mm | ~242mm x 112mm | ~242mm x 112mm |
| Power Connectors | 1x 16-pin (12VHPWR) | 1x 8-pin EPS | 1x 8-pin PCIe | 1x 12-pin or 2x 8-pin |
| Display Outputs | 1x HDMI 2.1, 3x DP 1.4a | 4x DisplayPort 1.4 | 1x HDMI 2.1, 3x DP 1.4a | 1x HDMI 2.1, 3x DP 1.4a |
| Max Resolution | 8K@60Hz | 4x 5K@60Hz | 4K@120Hz | 4K@120Hz |
| NVLink Support | No | Yes (3rd Gen, 112.5 GB/s) | No | No |
| ECC Memory | No | Yes | No | No |
| Multi-GPU Support | Yes (no NVLink) | Yes (with NVLink) | Yes (no NVLink) | Yes (no NVLink) |
| Target Market | Gaming/Enthusiast/Creator | Professional Workstation | Gaming/Mid-range | Gaming/Upper Mid-range |
| Launch Date | October 2022 | October 2020 | February 2021 | October 2020 |

DigitalOcean GPU Droplets - On Demand Pricing:

DigitalOcean GPU Droplets – On-Demand Pricing

| GPU Model | Price (per GPU/hour) | GPUs per Droplet | GPU VRAM | System RAM | vCPUs | Boot Disk | Outbound Transfer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NVIDIA RTX 6000 Ada Generation | $1.57 | 1 | 48 GB | 64 GiB | 8 | 500 GiB NVMe | 10,000 GiB |
| NVIDIA L40S | $1.57 | 1 | 48 GB | 64 GiB | 8 | 500 GiB NVMe | 10,000 GiB |

VRAM increases when running models together due to shared CUDA/cuDNN workspaces, memory fragmentation, batch/input scaling, and non-linear framework overhead — computational cost may change, this is just an estimated cost.